BedJetWebSchedule by Dan Roberts
User Guide

ESP32 BedJet BLE controller with Web UI, Scheduler, and Config Portal

Version: v29 (Doc build: 2025-12-26)

# At a glance

| What it is | An ESP32-based controller that connects to a BedJet over Bluetooth Low Energy (BLE) and exposes a web interface for manual control and scheduled automation. |
| --- | --- |
| Primary access | Web UI at the device IP address (e.g., http://192.168.x.x/) or mDNS hostname (default: http://bedjetweb.local/). |
| Setup mode | If not configured (or forced), the ESP32 starts an AP (BedJetSetup-XXXX) at http://192.168.4.1/ to capture Wi-Fi, BedJet MAC, and network settings. |
| Scheduling | On-device scheduler applies actions at configured start/stop times (mode + fan + temperature + runtime). |
| Persistence | Configuration and schedules are stored in non-volatile storage (NVS) and survive reboots/power loss. |
| Best practice | Place ESP32 within good RF range of both the BedJet and your Wi-Fi AP (2.4 GHz). Disable Wi-Fi sleep is enabled in firmware for BLE stability. |

# Overview

BedJetWebSchedule provides two operating modes:
1) Setup Mode (Access Point) for initial provisioning.
2) Normal Mode on your home network with the full Web UI.
In Normal Mode, the ESP32 can connect to your BedJet via BLE, provide quick manual controls, and run scheduled actions automatically.

## Security and privacy notes

This project is intended for use on a trusted home network. Wi-Fi credentials and BedJet settings are stored in ESP32 NVS and are not encrypted by default. Treat the device like any other IoT appliance: keep it on a trusted SSID/VLAN, use strong Wi-Fi security (WPA2/WPA3), and do not expose the web interface directly to the public internet.

# Requirements

ESP32 (tested with ESP32-C6 and ESP32 WROOM variants depending on your build environment).

BedJet 3 with BLE enabled/available.

2.4 GHz Wi-Fi network (ESP32 and BedJet BLE share 2.4 GHz spectrum).

A browser (desktop or phone).

# First-time setup (AP provisioning)

If the device does not have a saved Wi-Fi configuration (or if you force setup), it starts a temporary Wi-Fi access point:
- SSID: BedJetSetup-XXXX
- Default portal URL: http://192.168.4.1/

## Step-by-step

Power on (or reset) the ESP32.

On your phone or laptop, join the Wi-Fi network BedJetSetup-XXXX.

Open a browser and navigate to http://192.168.4.1/.

Enter your Wi-Fi SSID and password.

Enter the BedJet MAC address (format: AA:BB:CC:DD:EE:FF).

Choose DHCP (recommended) or Static IP and fill network fields if using static addressing.

Set the Hostname (default: BEDJETWEB). This controls the DHCP hostname and mDNS name (bedjetweb.local).

Click Save & Reboot.

After reboot: rejoin your home Wi-Fi network. The ESP32 will come up on your LAN.

## After reboot: how to find the device

Preferred: open http://bedjetweb.local/ (mDNS).

Fallback: use the IP shown on Serial monitor or your router's DHCP client list (look for hostname BEDJETWEB).

If your phone cannot resolve .local: use the IP address directly.

# Normal mode Web UI

In Normal Mode, the main page provides:
- Connection controls (Connect / Disconnect / Refresh)
- Quick Controls (OFF, HEAT, TURBO, COOL, DRY, EXT-HEAT)
- Quick parameters (fan step, temperature setpoint, optional runtime hours/minutes)
- Scheduler table for time-of-day automation
- Import/Export schedule
- Status telemetry view (mode, temps, fan, remaining time) when connected and subscribed to notifications.

## Connect / Disconnect / Refresh

Connect establishes a BLE session to the BedJet and subscribes to status notifications. Disconnect terminates BLE to free the BedJet for other clients. Refresh updates UI state from the ESP32.

## Quick Controls

Quick Controls are one-touch actions. Each button can apply a sequence:
1) Ensure BLE is connected (using the same retry logic as Connect).
2) Send the selected mode button command.
3) Optionally set temperature.
4) Optionally set fan speed.
5) Optionally set a runtime timer (hours/minutes).

Quick parameters

| Field | Range | Notes |
| --- | --- | --- |
| Fan step | 0-19 | Represents 5-100% in 5% steps (step 0 = 5%, step 19 = 100%). |
| Temperature (F) | 66-104 | BedJet setpoint in degrees Fahrenheit (limits enforced by firmware). |
| Runtime (H:M) | 0-11 hours, 0-59 minutes | 0:00 means 'do not change timer' for the command. |

# Scheduler

The scheduler runs on the ESP32 and applies actions based on local time. Schedules are stored in NVS and continue to execute after reboot.

## Schedule fields

| Enabled | Start (HH:MM) | Stop (HH:MM) | Mode | Temp (F) | Fan step |
| --- | --- | --- | --- | --- | --- |

Each schedule row defines a start time and stop time. At start, the selected mode + parameters are applied. At stop, the schedule can either turn OFF or apply a stop action depending on your current firmware settings.

## Import and Export schedule

Export downloads the current schedule as JSON.

Import (Replace) overwrites the stored schedule with the provided JSON.

Tip: Keep a backup of your JSON exports when iterating on schedules.

# Configuration page (Normal Mode)

The /config page allows you to change the same provisioning settings without entering AP mode. Typical use cases include updating the BedJet MAC, switching DHCP vs Static IP, or changing the hostname.

| Setting | Where | Impact |
| --- | --- | --- |
| Wi-Fi SSID/Password | Setup AP and /config | Requires reboot to re-associate to the Wi-Fi network. |
| BedJet MAC | Setup AP and /config | Used by the BLE resolver and fallback connect attempts. |
| DHCP vs Static | Setup AP and /config | Static IP requires correct gateway/mask/DNS for your LAN. |
| Hostname | Setup AP and /config | Sets DHCP hostname and mDNS service (bedjetweb.local by default). |

# Troubleshooting

## Cannot open the web page (timeout)

Confirm your phone or PC is on the same LAN/VLAN as the ESP32.

If you just came from Setup AP: your phone may still be attached to the old AP or switching networks. Re-join your home SSID explicitly.

Check your router DHCP list for the device hostname (BEDJETWEB).

If Wi-Fi association fails, the firmware is designed to fall back to Setup AP after a connect timeout.

## BLE connect fails or is unreliable

Keep the ESP32 physically close to the BedJet for testing (same room, 6-10 feet).

Avoid USB 3.0 hubs/cables near the ESP32 (they can produce 2.4 GHz interference).

If logs show 'scan: 0 results', the BedJet may be advertising infrequently. Try again or interact with the BedJet (button press) and reconnect.

Wi-Fi and BLE share 2.4 GHz; heavy Wi-Fi traffic can reduce scan reliability. Increase the UI refresh interval if needed.

## Phone cannot resolve bedjetweb.local

Many Android devices/networks do not resolve .local by default. Use the IP address instead.

Alternatively, install an mDNS resolver app on the phone or rely on router DHCP hostname lists.

# Recovery / Factory reset

If you lose access due to a bad network configuration, you can force Setup AP mode on boot. Depending on your hardware, holding the BOOT button (GPIO0) during reset/power-on may force setup mode. Use this to re-enter Wi-Fi and IP settings.

# Reference: common URLs

Main UI (IP): http://<device-ip>/

Main UI (mDNS): http://bedjetweb.local/

Config page: http://<device-ip>/config

Setup portal (AP mode): http://192.168.4.1/

End of document.