BedJetWebSchedule by Dan Roberts
Developer Guide

Architecture, build, configuration, and extension guide for contributors

Version: v29 (Doc build: 2025-12-26)

# 1. Project overview

BedJetWebSchedule is an ESP32 firmware application that controls a BedJet over BLE, exposes a web interface for manual controls and schedule management, and stores configuration and schedule state in non-volatile storage (NVS). The project is intended to be published on GitHub and extended by contributors.

## Goals

Reliable BLE connection and notification subscription to BedJet status characteristic.

Fast, mobile-friendly Web UI for manual control and schedule editing.

On-device scheduler that applies actions at configured times without requiring an always-on client.

Safe persistence model: config + schedules survive reboot; recovery via AP provisioning mode.

Contributor-friendly: clear module boundaries, explicit command encoding, and serial logging.

## Non-goals

Cloud dependency (the firmware runs fully offline on LAN).

Public internet exposure (no auth is included by default).

Full BedJet protocol reimplementation (only the commonly used commands are supported).

# 2. Repository layout (recommended)

The current implementation is a single .ino sketch for ease of Arduino IDE use. For GitHub and collaboration, consider splitting into .h/.cpp modules. A recommended layout:

src/

main.ino or main.cpp

webui.h / webui.cpp        (HTML/JS, handlers, gzip serving)

ble.h / ble.cpp            (connect, resolve, notify parse, command write)

scheduler.h / scheduler.cpp (schedule data model, evaluation loop, persistence)

config.h / config.cpp      (NVS config, AP portal, /config page)

util.h / util.cpp          (logging, parsing, html escaping)

include/ (optional)

data/ (optional - source HTML before gzip)

LICENSE

README.md

docs/ (this guide, user guide, protocol notes)

# 3. Build and toolchain

Supported environments: Arduino IDE 2.x and PlatformIO (recommended for CI).

## Arduino IDE

Install ESP32 core (tested with esp32 3.3.x).

Install NimBLE-Arduino library compatible with your ESP32 core.

Select correct board (ESP32 WROOM / ESP32-C6, etc.).

Set Flash size/partition to support larger sketches if available (e.g., 'Huge APP' if your board supports it).

Compile and upload via USB.

## PlatformIO

PlatformIO is recommended for reproducible builds. Define framework=arduino, platform=espressif32, and pin library versions. Use CI to enforce compilation on at least one reference target.

# 4. Runtime modes

| Mode | Trigger | Behavior |
| --- | --- | --- |
| Setup AP Mode | No saved config OR forced setup button (GPIO0/BOOT) | Starts softAP, hosts provisioning portal at 192.168.4.1, saves config to NVS, reboots. |
| Normal Mode | Valid config present | Joins Wi-Fi STA, starts WebServer, mDNS, scheduler loop, optional BLE on demand. |

# 5. Configuration storage (NVS)

Configuration is stored in NVS (Preferences) under a dedicated namespace (e.g., "cfg"). Schedules are stored separately (e.g., namespace "sched" or "bedjet") so config changes do not wipe schedules.

## Config keys (typical)

| Key | Type | Description |
| --- | --- | --- |
| wifi_ssid | string | Home Wi-Fi SSID |
| wifi_pass | string | Home Wi-Fi password |
| bedjet_mac | string | Target BedJet MAC |
| dhcp | bool | DHCP enabled (true) or static (false) |
| ip | string | Static IP (if dhcp=false) |
| gw | string | Gateway |
| mask | string | Subnet mask |
| dns | string | DNS server |
| hostname | string | DHCP hostname and mDNS service (default BEDJETWEB) |

# 6. Web server and UI

The project uses the synchronous Arduino WebServer. Because this server can be effectively single-client under load, responses are sent with 'Connection: close' and sockets are closed after responses. The UI is embedded in flash; to stay within flash limits the main page may be stored as a gzip-compressed byte array served with 'Content-Encoding: gzip'.

## Key endpoints

| Endpoint | Method | Purpose |
| --- | --- | --- |
| / | GET | Main UI page (possibly gzipped). |
| /api/state | GET | Return current device state + last BedJet status frame decoded. |
| /api/ble/connect | POST/GET | Initiate BLE connect sequence with retries. |
| /api/ble/disconnect | POST/GET | Disconnect BLE. |
| /api/cmd/button | POST/GET | Quick control button press; may include temp/fan/timer parameters. |
| /api/schedule/export | GET | Export schedule JSON. |
| /api/schedule/import | POST | Import schedule JSON (Replace). |
| /config | GET/POST | Normal-mode config view/edit (same keys as AP portal). |

## UI behavior

Buttons show 'pressed' feedback and disable while a request is in-flight.

Modal dialogs are used for connect and quick actions, showing progress and result.

UI polling is paused when the tab is hidden (document.hidden) to reduce Wi-Fi load.

Use conservative UI refresh intervals to improve BLE scan reliability on 2.4 GHz coexistence.

# 7. BLE layer

The BLE layer uses NimBLE-Arduino and implements:
- Discovery/resolve via scan (match MAC/service/name)
- Connect with retry and client reset
- Service/characteristic discovery
- Subscribe to status notifications
- Encode and write command packets to the command characteristic

## BedJet BLE UUIDs

| UUID | Role | Notes |
| --- | --- | --- |
| 00001000-bed0-0080-aa55-4265644a6574 | Primary service | BedJet custom service |
| 00002000-bed0-0080-aa55-4265644a6574 | Status notify | 20-byte status frames |
| 00002001-bed0-0080-aa55-4265644a6574 | Device name | Readable characteristic |
| 00002004-bed0-0080-aa55-4265644a6574 | Command write | Write commands/buttons/timer/temp/fan |
| 00002005-bed0-0080-aa55-4265644a6574 | Version A | Readable |
| 00002006-bed0-0080-aa55-4265644a6574 | Version B | Readable |

## Status frame parsing

Status notifications are 20 bytes. Common fields observed in reverse engineered implementations:
- [4],[5],[6] remaining time (hours/minutes/seconds)
- [7] actual temperature byte
- [8] set temperature byte
- [9] mode index (lookup: off, heat, turbo, ext-heat, cool, dry)
- [10] fan step index (lookup to percent)
Implementations may convert temp bytes to Fahrenheit; be consistent across UI and scheduler.

## Command encoding

Commands are written to 0x2004 as small byte arrays. Known patterns:
- Buttons/modes: [0x01, <buttonCode>]
- Set timer: [0x02, hours, minutes]
- Set temperature: [0x03, tempByte]
- Set fan: [0x07, fanIndex]
Use short inter-command delays (e.g., 60-120ms) to improve reliability.

# 8. Scheduler

The scheduler evaluates every second (or similar cadence) and determines whether any schedule transitions occur. At start time: apply configured mode/temp/fan/runtime. At stop time: typically send OFF or a stop action.
The scheduler must avoid interfering with user-initiated actions by using a shared 'BLE busy' guard.

## Schedule data model

ScheduleItem typically includes:
- id (uint16)
- enabled (bool)
- startMin, stopMin (minutes since midnight)
- mode/button enum
- tempF (int)
- fanStep (0-19)
- runtimeH/runtimeM (optional)
Persist schedule list to NVS in a compact representation.

# 9. mDNS and hostname

The firmware sets a DHCP hostname (for router client lists) and starts mDNS so clients can browse to http://<hostname>.local/. Because Android .local support is inconsistent, always display and log the IP address.

# 10. Performance and reliability considerations

Avoid blocking loops in setup() (e.g., waiting indefinitely for NTP); start services early.

Prefer short repeated scans over one long scan if Wi-Fi traffic is high.

Close HTTP connections after each response to prevent single-client starvation with WebServer.

Disable Wi-Fi sleep to reduce BLE scan/connect jitter: WiFi.setSleep(false).

Throttle UI polling and pause when tab hidden to reduce 2.4 GHz contention.

# 11. Flash size management

If the sketch approaches flash limits, gzip compress large HTML/JS assets and serve them with Content-Encoding: gzip. Keep string literals in PROGMEM and avoid duplicate copies of UI strings. Consider moving JSON import parsing to a small library if it reduces code size overall.

# 12. Contributing guidelines

Keep changes modular; avoid touching BLE internals when working on UI and vice versa.

Add Serial logging for connect/scan stages and for schedule triggers.

Prefer small, testable changes; include before/after notes in PRs.

Document any new schedule fields and update both User Guide and Developer Guide.

## Suggested commit message patterns

feat(web): add modal progress for quick actions

fix(ble): improve scan resolve by extending scan window and retry

refactor(cfg): isolate NVS keys and add /config editor

perf(ui): gzip index.html and serve with Content-Encoding header

docs: add developer guide and update endpoint reference

# 13. Appendix: Troubleshooting for developers

## Compilation / Arduino preprocessor issues

Arduino auto-generates prototypes at the top of the sketch. If a function signature uses a type defined later (e.g., ScheduleItem), add a forward declaration near the top: 'struct ScheduleItem;'. Alternatively, move the type definition above the first function or migrate to .h/.cpp modules.

## Multi-client web UI timeouts

Synchronous WebServer can starve multiple clients. Ensure:
- Connection: close headers
- server.client().stop() after sending
- UI throttles polling and pauses on background tabs

## BLE scan returns 0 results

This indicates no advertisements were observed during the scan window. Increase scan window, repeat scans, reduce Wi-Fi activity, and test closer proximity. BedJet may advertise at low duty cycle when idle.

End of document.